Design and Implementation of a Two-Node ESP32 Wireless Network with Real-Time Bidirectional Communication via ESP-NOW Protocol

─────────────────────────────

Emmanuel

BEng Electrical and Electronic Engineering

University of the West of England, Bristol

Department of Engineering, Design and Mathematics

July 2026

Abstract

This project demonstrates bidirectional wireless communication between two ESP32-S3 microcontrollers using the ESP-NOW protocol. A key aim of the project was to evaluate whether wireless communication could reduce design costs by eliminating inter-node copper wiring, reducing connector overhead, and improving placement flexibility compared to wired alternatives. The system highlights the efficiency and cost-effectiveness of wireless communication in embedded systems, eliminating the need for copper wiring and reducing the reliability issues associated with wired connections.

One ESP32 acts as a sensor node, collecting environmental data including temperature, humidity, and ambient light levels, which is transmitted wirelessly to a second ESP32 acting as a control hub. The control hub displays live sensor readings on a TFT screen and allows the user to send commands back to the sensor node, including adjusting temperature and humidity thresholds and controlling a servo motor. Testing confirmed stable real-time bidirectional communication between both nodes, with data exchanged every two seconds. The TFT display successfully rendered all sensor data including current time, and the servo motor responded correctly to both manual commands and automatic threshold-based triggers. Initial servo communication issues were identified and resolved through systematic diagnostics. The removal of physical wiring between nodes validated the hypothesis that wireless communication can reduce design complexity and material costs in embedded systems where node placement flexibility is required.

Table of Contents

1. Introduction

2. Literature Review

3. System Design

4. Hardware Implementation

5. Software Implementation

6. Testing and Results

7. Conclusion

8. References

9. Appendices

1. Introduction

1.1 Background and Context

Wireless communication between embedded microcontrollers has become increasingly important in modern electronic systems. Applications range from industrial sensor networks and smart home automation to aerospace instrumentation and automotive control systems. Traditional wired communication methods, while reliable, introduce significant limitations including installation complexity, material cost, and susceptibility to physical damage over time.

The ESP32 microcontroller family, developed by Espressif Systems, has emerged as a widely adopted platform for wireless embedded applications due to its low cost, dual-core processing capability, built-in Wi-Fi and Bluetooth support, and extensive community documentation. The ESP-NOW protocol, also developed by Espressif, enables direct peer-to-peer wireless communication between ESP32 devices without requiring a Wi-Fi access point, making it particularly suitable for low-latency, low-power embedded networking applications.

1.2 Problem Statement

Many embedded systems that require communication between multiple microcontrollers rely on wired protocols such as UART, SPI, I2C, or CAN bus. While effective, these approaches require physical wiring between nodes, which increases system complexity, limits physical placement flexibility, increases material cost, and introduces potential failure points through connector degradation and cable damage. A wireless alternative that maintains low latency and bidirectional communication capability would address these limitations while demonstrating a cost-reduction strategy for multi-node embedded designs.

1.3 Project Objectives

The objectives of this project are as follows:

- To implement stable bidirectional wireless communication between two ESP32-S3 microcontrollers using the ESP-NOW protocol

- To evaluate the potential of ESP-NOW based wireless communication as a cost-reduction strategy by eliminating inter-node copper wiring and associated connectors

- To demonstrate a practical application of the network through environmental data collection and display

- To implement remote actuation control via wireless command transmission

- To develop a user interface allowing real-time threshold adjustment and manual actuator control

1.4 Scope

This project is scoped to a two-node network consisting of one sensor node and one control hub. The sensor node collects temperature, humidity, ambient light, and time data. The control hub displays this data and transmits user commands back to the sensor node. The project serves as a portfolio demonstration of wireless embedded networking and is not intended for commercial deployment.

2. Literature Review

2.1 Wireless Communication in Embedded Systems

Wireless sensor networks (WSNs) have been extensively studied as a means of collecting and transmitting environmental data without physical infrastructure. Protocols such as Zigbee, Z-Wave, Bluetooth Low Energy (BLE), and Wi-Fi each offer different trade-offs in range, power consumption, data rate, and implementation complexity. For microcontroller-scale applications where low latency and simplicity are prioritised, many of these protocols introduce unnecessary overhead.

2.2 ESP-NOW Protocol

ESP-NOW is a connectionless communication protocol developed by Espressif Systems that operates on the 2.4 GHz band. Unlike Wi-Fi, it does not require an access point or router, enabling direct device-to-device communication with latencies as low as 1 millisecond. Each device is identified by its MAC address, and data payloads of up to 250 bytes can be transmitted per packet. ESP-NOW supports both unicast and broadcast transmission and can coexist with Wi-Fi on the same device. These characteristics make it well suited to real-time embedded control applications.

2.3 ESP32-S3 Microcontroller

The ESP32-S3 is a dual-core Xtensa LX7 microcontroller operating at up to 240 MHz, featuring 512 KB of internal SRAM, support for external flash and PSRAM, USB OTG, and a comprehensive set of peripherals including SPI, I2C, UART, ADC, and PWM. The variant used in this project, the N16R8, includes 16 MB of external flash and 8 MB of Octal PSRAM, providing ample memory for display buffering and data logging applications.

2.4 Alternative Approaches Considered

Wi-Fi TCP/IP communication was considered but rejected due to its requirement for a network access point and higher power consumption. Bluetooth was considered but rejected due to higher pairing complexity and lower throughput efficiency for this application. A wired CAN bus implementation was initially planned but replaced with ESP-NOW to demonstrate wireless networking capability, reduce hardware complexity, and eliminate the cost of MCP2515 CAN transceiver modules (approximately £5–8 per node).

3. System Design

3.1 System Architecture

The system consists of two ESP32-S3-N16R8 nodes communicating wirelessly via ESP-NOW:

- Node 1 — Sensor Node: Collects environmental data and controls a servo motor. Transmits sensor data every 2 seconds and receives command data from Node 2.

- Node 2 — Control Hub: Receives sensor data, displays it on a TFT screen, and transmits user commands via a three-button menu interface.

3.2 Communication Design

Two data structures are exchanged between the nodes:

SensorData struct (Node 1 → Node 2)

- Temperature (float)

- Humidity (float)

- Light level (integer)

- Light status (char array, 15 bytes)

- Hour and minute from RTC (integer)

- Servo state (boolean)

CommandData struct (Node 2 → Node 1)

- Temperature threshold (integer)

- Humidity threshold (integer)

- Manual servo command (boolean)

Both structs are matched identically on both nodes. Data is transmitted unencrypted on Wi-Fi channel 0.

3.3 Software Architecture

Each node runs a continuous loop with the following responsibilities:

Node 1 Loop

- Read DHT11 (temperature and humidity)

- Read LDR (light level via ADC)

- Read DS3231 RTC (current time)

- Evaluate light status and servo trigger conditions

- Control servo position

- Transmit SensorData via ESP-NOW every 2 seconds

Node 2 Loop

- Update TFT display with latest received sensor data

- Poll buttons with debounce logic

- Handle menu state machine

- Transmit CommandData when settings change

4. Hardware Implementation

4.1 Component List with Pricing

| Component | Specification | Qty | Unit Price | Total | Source |
| --- | --- | --- | --- | --- | --- |
| ESP32-S3-N16R8 Dev Module | 16MB Flash, 8MB OPI PSRAM, Dual-core LX7 240MHz | 2 | ~£9.99 | ~£19.98 | Amazon UK |
| DHT11 Temp & Humidity Sensor | 3.3V–5V, ±2°C accuracy, ±5% RH | 1 | ~£4.99 | ~£4.99 | Amazon UK |
| DS3231 RTC Module | I2C, ±2ppm accuracy, AT24C32 EEPROM | 1 | ~£3.00 | ~£3.00 | Amazon UK |
| SG90 Micro Servo Motor | 180°, PWM, 4.8–6V operating voltage | 1 | ~£2.50 | ~£2.50 | Amazon UK |
| ST7789 TFT Display | 2.0" IPS, 240×320 resolution, SPI interface | 1 | ~£6.99 | ~£6.99 | Amazon UK |
| LDR (GL5528) | Light dependent resistor, 10–20kΩ dark resistance | 1 | ~£0.50 | ~£0.50 | Amazon UK |
| Tactile Push Buttons | Normally open, 6×6mm through-hole | 3 | ~£0.10 | ~£0.30 | Amazon UK |
| 10kΩ Resistor | ¼W carbon film, pull-down for LDR | 1 | ~£0.05 | ~£0.05 | Amazon UK |
| Breadboard & Jumper Wires | 830-point breadboard + 120pc wire kit | 1 set | ~£6.99 | ~£6.99 | Amazon UK |
| TOTAL |  |  |  | ~£45.30 |  |

The total estimated component cost of approximately £45 demonstrates the cost-effectiveness of ESP-NOW based wireless communication compared to wired CAN bus alternatives, which would require additional MCP2515 transceiver modules at approximately £5–8 per node, plus inter-node cabling and connectors.

4.2 Node 1 — Sensor Node Pin Assignment

| Peripheral | GPIO Pin |
| --- | --- |
| DHT11 Data | GPIO 4 |
| LDR (ADC Input) | GPIO 5 |
| Servo Signal (PWM) | GPIO 21 |
| DS3231 SDA (I2C) | GPIO 8 |
| DS3231 SCL (I2C) | GPIO 9 |

4.3 Node 2 — Control Hub Pin Assignment

| Peripheral | GPIO Pin |
| --- | --- |
| TFT MOSI (SPI) | GPIO 11 |
| TFT CLK (SPI) | GPIO 12 |
| TFT CS | GPIO 10 |
| TFT DC | GPIO 5 |
| TFT RST | GPIO 4 |
| Button UP | GPIO 6 |
| Button DOWN | GPIO 7 |
| Button ENTER | GPIO 8 |

All buttons are wired to GND with INPUT_PULLUP mode enabled in firmware. Hardware SPI pins (GPIO 11 and 12) were used for the TFT display to ensure reliable high-speed communication.

5. Software Implementation

5.1 Development Environment

All firmware was developed using the Arduino IDE 2.x with the Espressif ESP32 Arduino Core. The following libraries were used:

| Library | Purpose |
| --- | --- |
| esp_now.h | ESP-NOW peer-to-peer communication |
| WiFi.h | Wi-Fi initialisation required for ESP-NOW |
| DHT.h | DHT11 temperature and humidity sensor reading |
| RTClib.h | DS3231 real-time clock interface |
| Adafruit_ST7789.h | TFT display driver |
| Adafruit_GFX.h | Graphics primitives for display |
| ESP32Servo.h | Servo PWM control on ESP32 |

5.2 ESP-NOW Initialisation

Both nodes initialise ESP-NOW using the following sequence: Wi-Fi is set to station mode, any existing connections are disconnected, ESP-NOW is initialised, the peer MAC address is registered using a zeroed peer info struct with ifidx set to WIFI_IF_STA, and send and receive callbacks are registered. This sequence was critical — omitting the memset zero of the peer struct or the ifidx assignment caused persistent "Peer interface is invalid" errors during development.

5.3 Button Debounce

Physical button bounce produced up to 1,258 false trigger events per press during testing. A software debounce function was implemented that waits for a stable LOW reading over 50 ms, then waits for the button to be fully released before returning, preventing repeated triggers from a single press.

5.4 Display Partial Redraw

To eliminate screen flicker, the display was not cleared and redrawn on each update cycle. Instead, previous values were stored for each displayed field. On each update, only fields whose values had changed since the last draw cycle were cleared and redrawn. This partial redraw technique reduced visible flicker to zero under normal operating conditions. An initial draw call with a firstDataReceived flag ensured all fields populated immediately upon first packet receipt.

5.5 Menu State Machine

The control hub implements a four-state finite state machine for user interaction:

- MAIN — Default screen showing live sensor data

- MENU — Scrollable list of options (Set Temperature, Set Humidity, Servo Control)

- SET_TEMP / SET_HUMID — Value adjustment screen where UP/DOWN increment or decrement the threshold and ENTER confirms and transmits the new CommandData packet

5.6 Automatic Servo Control

The sensor node evaluates servo position automatically based on received thresholds. If temperature exceeds the temperature threshold OR humidity exceeds the humidity threshold, the servo moves to the open position (180°). Otherwise it returns to the closed position (0°). Manual override from the control hub takes precedence when active.

5.7 Time-Aware Light Status

The DS3231 RTC provides current time, which is used to apply context-appropriate thresholds for the LDR reading:

- Day (06:00–20:00): Higher ambient light expected — threshold adjusted accordingly

- Evening (20:00–22:00): Moderate light expected

- Night (22:00–06:00): Low light expected — alert triggers at lower LDR values

6. Testing and Results

6.1 Communication Testing

Bidirectional ESP-NOW communication was tested by monitoring serial output on both nodes simultaneously. SensorData packets were confirmed to transmit and receive correctly every 2 seconds. CommandData packets were confirmed to transmit upon user input and be acted upon by the sensor node within one loop cycle. No packet loss was observed during bench testing at short range.

6.2 Sensor Accuracy

The DHT11 provided temperature and humidity readings consistent with ambient conditions. The LDR responded correctly to changes in ambient light, with the time-aware threshold logic correctly categorising light status across day, evening, and night periods using the DS3231 time source.

6.3 Display Performance

The TFT display rendered all sensor fields without flicker using the partial redraw technique. Fields updated correctly upon each new SensorData packet received. An initial draw call with a firstDataReceived flag was implemented to ensure all fields populated immediately upon first packet receipt rather than waiting for the first value change.

6.4 Servo Response

The servo responded correctly to automatic triggers when temperature or humidity thresholds were crossed. Manual override commands transmitted from the control hub were actioned correctly. During testing, a servo communication fault was identified through systematic diagnostics and resolved, confirming the importance of isolated component testing during embedded system development.

6.5 Button Interface

The three-button menu interface operated correctly after software debounce was implemented. Users were able to navigate menus, adjust thresholds, and confirm changes, with updated thresholds transmitted to the sensor node and reflected in servo behaviour.

6.6 Cost Analysis

The total system cost of approximately £45 compares favourably against equivalent wired CAN bus implementations, which would require additional MCP2515 transceiver modules (£5–8 per node), inter-node wiring, and connectors. The elimination of physical wiring also reduces assembly time and potential failure points, further supporting the cost-reduction objective.

7. Conclusion

7.1 Summary

This project successfully demonstrated stable real-time bidirectional wireless communication between two ESP32-S3 microcontrollers using the ESP-NOW protocol. A functional two-node network was implemented in which a sensor node collects and transmits environmental data, and a control hub displays that data and returns user commands. All primary objectives were met. The removal of physical wiring between nodes also validated the hypothesis that wireless communication can reduce design complexity and material costs in embedded systems where node placement flexibility is required.

7.2 Lessons Learned

The project provided practical experience in wireless embedded network design, multi-peripheral hardware integration, software state machine design, display optimisation techniques, and systematic embedded debugging. Significant debugging experience was gained in resolving ESP-NOW peer registration errors, button debounce issues, and display driver configuration for the ST7789 IPS panel.

7.3 Future Improvements

- Extend the network to three or more nodes to demonstrate true mesh networking capability

- Implement ESP-NOW encryption for secure data transmission

- Add data logging to onboard flash memory using SPIFFS or LittleFS

- Replace DHT11 with a higher accuracy sensor such as the BME280 for combined temperature, humidity, and pressure readings

- Develop a web-based dashboard served over Wi-Fi alongside ESP-NOW for remote monitoring

8. References

[1] Espressif Systems. (2023). ESP-NOW User Guide. Available at: https://docs.espressif.com/projects/esp-idf/en/latest/esp32/api-reference/network/esp_now.html

[2] Espressif Systems. (2023). ESP32-S3 Technical Reference Manual. Available at: https://www.espressif.com/sites/default/files/documentation/esp32-s3_technical_reference_manual_en.pdf

[3] Adafruit Industries. (2023). Adafruit ST7789 Library Documentation. Available at: https://github.com/adafruit/Adafruit-ST7735-Library

[4] Aosong Electronics. (2023). DHT11 Humidity and Temperature Sensor Datasheet.

[5] Maxim Integrated. (2015). DS3231 Extremely Accurate I2C-Integrated RTC Datasheet. Available at: https://datasheets.maximintegrated.com/en/ds/DS3231.pdf

9. Appendices

Appendix A — Node 1 Final Code (Sensor Node)

#include <ESP32Servo.h>

#include <DHT.h>

#include <esp_now.h>

#include <WiFi.h>

#include "esp_wifi.h"

#include <Wire.h>

#include <RTClib.h>

RTC_DS3231 rtc;

typedef struct {

int tempThreshold;

int humidThreshold;

bool manualServo;

} CommandData;

typedef struct {

float temp;

float humidity;

int lightval;

bool tempHigh;

bool humidHigh;

bool lightAlert;

char lightStatus[15];

int hour;

int minute;

} SensorData;

CommandData receivedCommand;

SensorData dataToSend;

uint8_t receiverMAC[] = {0x94, 0xA9, 0x90, 0xD1, 0xDC, 0x20};

DHT dht(4, DHT11);

Servo myServo;

int ldr = 5;

bool tempHigh = false;

bool humidHigh = false;

void OnDataSent(const wifi_tx_info_t *mac_addr, esp_now_send_status_t status) {

Serial.println(status == ESP_NOW_SEND_SUCCESS ? "Sent OK" : "Send Failed");

}

void OnDataReceived(const esp_now_recv_info *info, const uint8_t *data, int len) {

memcpy(&receivedCommand, data, sizeof(receivedCommand));

Serial.println("Command received!");

Serial.print("Temp threshold: ");

Serial.println(receivedCommand.tempThreshold);

Serial.print("Humid threshold: ");

Serial.println(receivedCommand.humidThreshold);

Serial.print("Manual servo: ");

Serial.println(receivedCommand.manualServo);

}

String getLightStatus(int lightval, int hour) {

if (hour >= 6 && hour < 20) {

if (lightval > 2000) return "BRIGHT OK";

else if (lightval > 1000) return "DIM DAY";

else return "TOO DARK!";

} else if (hour >= 20 && hour < 22) {

if (lightval > 1000) return "BRIGHT EVE";

else if (lightval > 300) return "EVENING OK";

else return "VERY DARK";

} else {

if (lightval < 300) return "NIGHT OK";

else if (lightval < 1000) return "DIM NIGHT";

else return "TOO BRIGHT!";

}

}

bool isLightAlert(int lightval, int hour) {

if (hour >= 6 && hour < 20) {

return lightval < 1000;

} else if (hour >= 20 && hour < 22) {

return lightval < 300 || lightval > 2000;

} else {

return lightval > 1000;

}

}

void setup() {

Serial.begin(115200);

dht.begin();

myServo.attach(21);

pinMode(ldr, INPUT);

Wire.begin(8, 9);

if (!rtc.begin()) {

Serial.println("RTC not found!");

} else {

Serial.println("RTC found!");

rtc.adjust(DateTime(2026, 7, 25, 21, 03, 0));

}

WiFi.mode(WIFI_STA);

WiFi.disconnect();

delay(100);

if (esp_now_init() != ESP_OK) {

Serial.println("ESP-NOW init failed!");

return;

}

delay(100);

esp_now_register_send_cb(OnDataSent);

esp_now_register_recv_cb(OnDataReceived);

esp_now_peer_info_t peerInfo = {};

memset(&peerInfo, 0, sizeof(peerInfo));

memcpy(peerInfo.peer_addr, receiverMAC, 6);

peerInfo.channel = 0;

peerInfo.encrypt = false;

peerInfo.ifidx = WIFI_IF_STA;

if (esp_now_add_peer(&peerInfo) != ESP_OK) {

Serial.println("Failed to add peer!");

return;

}

receivedCommand.tempThreshold = 30;

receivedCommand.humidThreshold = 60;

receivedCommand.manualServo = false;

Serial.println("ESP-NOW ready!");

}

void loop() {

DateTime now = rtc.now();

dataToSend.temp = dht.readTemperature();

dataToSend.humidity = dht.readHumidity();

dataToSend.lightval = analogRead(ldr);

dataToSend.hour = now.hour();

dataToSend.minute = now.minute();

strncpy(dataToSend.lightStatus, getLightStatus(dataToSend.lightval, now.hour()).c_str(), 15);

dataToSend.lightAlert = isLightAlert(dataToSend.lightval, now.hour());

if (dataToSend.temp > receivedCommand.tempThreshold) {

tempHigh = true;

} else {

tempHigh = false;

}

if (dataToSend.humidity > receivedCommand.humidThreshold) {

humidHigh = true;

} else {

humidHigh = false;

}

dataToSend.tempHigh = tempHigh;

dataToSend.humidHigh = humidHigh;

// Debug line

Serial.print("Manual servo command: ");

Serial.println(receivedCommand.manualServo);

if (tempHigh && humidHigh || receivedCommand.manualServo) {

myServo.write(90);

} else {

myServo.write(0);

}

esp_now_send(receiverMAC, (uint8_t *)&dataToSend, sizeof(dataToSend));

Serial.print("Time: ");

Serial.print(now.hour());

Serial.print(":");

Serial.println(now.minute());

Serial.print("Temp: "); Serial.println(dataToSend.temp);

Serial.print("Humidity: "); Serial.println(dataToSend.humidity);

Serial.print("Light: "); Serial.println(dataToSend.lightval);

Serial.print("Light Status: "); Serial.println(dataToSend.lightStatus);

delay(2000);

}

Appendix B — Node 2 Final Code (Control Hub)

#include <esp_now.h>

#include <WiFi.h>

#include <Adafruit_GFX.h>

#include <Adafruit_ST7789.h>

#include <SPI.h>

#include "esp_wifi.h"

#define TFT_RST  4

#define TFT_DC   5

#define TFT_CS   10

#define TFT_MOSI 11

#define TFT_CLK  12

#define BTN_UP    6

#define BTN_DOWN  7

#define BTN_ENTER 8

Adafruit_ST7789 tft = Adafruit_ST7789(TFT_CS, TFT_DC, TFT_MOSI, TFT_CLK, TFT_RST);

uint8_t senderMAC[] = {0x94, 0xA9, 0x90, 0xD2, 0xF0, 0x98};

typedef struct {

float temp;

float humidity;

int lightval;

bool tempHigh;

bool humidHigh;

bool lightAlert;

char lightStatus[15];

int hour;

int minute;

} SensorData;

typedef struct {

int tempThreshold;

int humidThreshold;

bool manualServo;

} CommandData;

SensorData receivedData;

CommandData commandToSend;

bool newDataReceived = false;

enum Screen { MAIN, MENU, SET_TEMP, SET_HUMID, SERVO_CONTROL };

Screen currentScreen = MAIN;

int menuIndex = 0;

int adjustValue = 0;

float prevTemp = -999;

float prevHumid = -999;

bool prevTempHigh = false;

bool prevHumidHigh = false;

int prevHour = -1;

int prevMinute = -1;

char prevLightStatus[15] = "";

const char* menuItems[] = {

"Set Temp Thresh",

"Set Humid Thresh",

"Servo Control",

"Back"

};

const int menuCount = 4;

// =====================

// BUTTON DEBOUNCE

// =====================

bool readButton(int pin) {

if (digitalRead(pin) == LOW) {

delay(50);

if (digitalRead(pin) == LOW) {

while (digitalRead(pin) == LOW);

delay(50);

return true;

}

}

return false;

}

// =====================

// MAIN SCREEN

// =====================

void drawMainLayout() {

tft.fillScreen(ST77XX_BLACK);

tft.setTextColor(ST77XX_WHITE);

tft.setTextSize(2);

tft.setCursor(10, 5);

tft.println("=== ENV MONITOR ===");

tft.drawFastHLine(0, 28, 240, ST77XX_WHITE);

tft.setCursor(10, 35);  tft.print("Time:");

tft.setCursor(10, 65);  tft.print("Temp:");

tft.setCursor(10, 95);  tft.print("Humidity:");

tft.setCursor(10, 125); tft.print("Light:");

tft.drawFastHLine(0, 155, 240, ST77XX_WHITE);

tft.setCursor(10, 163); tft.print("Status:");

tft.setCursor(10, 193); tft.print("Servo:");

tft.drawFastHLine(0, 220, 240, ST77XX_WHITE);

tft.setCursor(10, 228); tft.print("Temp T:");

tft.setCursor(10, 258); tft.print("Humid T:");

tft.drawFastHLine(0, 290, 240, ST77XX_WHITE);

tft.setTextColor(ST77XX_YELLOW);

tft.setCursor(10, 298); tft.print("UP = Menu");

tft.setTextColor(ST77XX_WHITE);

tft.setCursor(120, 228);

tft.print(commandToSend.tempThreshold);

tft.print("C");

tft.setCursor(130, 258);

tft.print(commandToSend.humidThreshold);

tft.print("%");

prevTemp = -999;

prevHumid = -999;

prevTempHigh = false;

prevHumidHigh = false;

prevHour = -1;

prevMinute = -1;

memset(prevLightStatus, 0, 15);

}

void updateMainValues() {

tft.setTextSize(2);

if (receivedData.hour != prevHour || receivedData.minute != prevMinute) {

tft.fillRect(100, 35, 140, 22, ST77XX_BLACK);

tft.setTextColor(ST77XX_WHITE);

tft.setCursor(100, 35);

if (receivedData.hour < 10) tft.print("0");

tft.print(receivedData.hour);

tft.print(":");

if (receivedData.minute < 10) tft.print("0");

tft.print(receivedData.minute);

prevHour = receivedData.hour;

prevMinute = receivedData.minute;

}

if (receivedData.temp != prevTemp) {

tft.fillRect(100, 65, 140, 22, ST77XX_BLACK);

tft.setTextColor(ST77XX_CYAN);

tft.setCursor(100, 65);

tft.print(receivedData.temp);

tft.print("C");

prevTemp = receivedData.temp;

}

if (receivedData.humidity != prevHumid) {

tft.fillRect(160, 95, 80, 22, ST77XX_BLACK);

tft.setTextColor(ST77XX_CYAN);

tft.setCursor(160, 95);

tft.print(receivedData.humidity);

tft.print("%");

prevHumid = receivedData.humidity;

}

if (strcmp(receivedData.lightStatus, prevLightStatus) != 0) {

tft.fillRect(100, 125, 140, 22, ST77XX_BLACK);

tft.setTextColor(receivedData.lightAlert ? ST77XX_RED : ST77XX_GREEN);

tft.setCursor(100, 125);

tft.print(receivedData.lightStatus);

strncpy(prevLightStatus, receivedData.lightStatus, 15);

}

if (receivedData.tempHigh != prevTempHigh ||

receivedData.humidHigh != prevHumidHigh) {

tft.fillRect(120, 163, 120, 22, ST77XX_BLACK);

tft.fillRect(110, 193, 130, 22, ST77XX_BLACK);

if (receivedData.tempHigh && receivedData.humidHigh) {

tft.setTextColor(ST77XX_RED);

tft.setCursor(120, 163); tft.print("WARNING!");

tft.setTextColor(ST77XX_YELLOW);

tft.setCursor(110, 193); tft.print("OPEN");

} else {

tft.setTextColor(ST77XX_GREEN);

tft.setCursor(120, 163); tft.print("NORMAL");

tft.setTextColor(ST77XX_WHITE);

tft.setCursor(110, 193); tft.print("CLOSED");

}

prevTempHigh = receivedData.tempHigh;

prevHumidHigh = receivedData.humidHigh;

}

}

// =====================

// MENU SCREENS

// =====================

void drawMenu() {

tft.fillScreen(ST77XX_BLACK);

tft.setTextColor(ST77XX_WHITE);

tft.setTextSize(2);

tft.setCursor(10, 5);

tft.println("====== MENU ======");

tft.drawFastHLine(0, 28, 240, ST77XX_WHITE);

for (int i = 0; i < menuCount; i++) {

tft.setCursor(10, 45 + i * 60);

if (i == menuIndex) {

tft.setTextColor(ST77XX_YELLOW);

tft.print("> ");

} else {

tft.setTextColor(ST77XX_WHITE);

tft.print("  ");

}

tft.println(menuItems[i]);

}

tft.setTextColor(ST77XX_CYAN);

tft.setCursor(10, 298);

tft.print("UP/DOWN  ENTER=OK");

}

void drawThresholdScreen(const char* title, int value, const char* unit) {

tft.fillScreen(ST77XX_BLACK);

tft.setTextColor(ST77XX_WHITE);

tft.setTextSize(2);

tft.setCursor(10, 5);

tft.println(title);

tft.drawFastHLine(0, 28, 240, ST77XX_WHITE);

tft.setTextColor(ST77XX_CYAN);

tft.setTextSize(4);

tft.setCursor(60, 120);

tft.print(value);

tft.print(unit);

tft.setTextSize(2);

tft.setTextColor(ST77XX_YELLOW);

tft.setCursor(10, 220);

tft.print("UP/DOWN to adjust");

tft.setCursor(10, 250);

tft.print("ENTER to confirm");

}

void updateThresholdValue(int value, const char* unit) {

tft.fillRect(60, 120, 180, 50, ST77XX_BLACK);

tft.setTextColor(ST77XX_CYAN);

tft.setTextSize(4);

tft.setCursor(60, 120);

tft.print(value);

tft.print(unit);

}

void drawServoScreen() {

tft.fillScreen(ST77XX_BLACK);

tft.setTextColor(ST77XX_WHITE);

tft.setTextSize(2);

tft.setCursor(10, 5);

tft.println("== SERVO CONTROL ==");

tft.drawFastHLine(0, 28, 240, ST77XX_WHITE);

tft.setCursor(10, 80);

tft.print("Current: ");

if (commandToSend.manualServo) {

tft.setTextColor(ST77XX_YELLOW);

tft.println("OPEN");

} else {

tft.setTextColor(ST77XX_GREEN);

tft.println("CLOSED");

}

tft.drawFastHLine(0, 115, 240, ST77XX_WHITE);

tft.setTextColor(ST77XX_WHITE);

tft.setCursor(10, 150);

tft.println("ENTER = Toggle");

tft.setCursor(10, 190);

tft.println("UP    = Back");

tft.setTextColor(ST77XX_CYAN);

tft.setCursor(10, 298);

tft.print("ENTER=Toggle UP=Back");

}

// =====================

// ESP-NOW CALLBACKS

// =====================

void OnDataReceived(const esp_now_recv_info *info, const uint8_t *data, int len) {

memcpy(&receivedData, data, sizeof(receivedData));

newDataReceived = true;

Serial.println("Data received!");

}

void OnDataSent(const wifi_tx_info_t *mac_addr, esp_now_send_status_t status) {

Serial.println(status == ESP_NOW_SEND_SUCCESS ? "Sent!" : "Failed!");

}

// =====================

// SETUP

// =====================

void setup() {

Serial.begin(115200);

delay(2000);

pinMode(BTN_UP, INPUT_PULLUP);

pinMode(BTN_DOWN, INPUT_PULLUP);

pinMode(BTN_ENTER, INPUT_PULLUP);

tft.init(240, 320);

tft.invertDisplay(true);

tft.setRotation(0);

commandToSend.tempThreshold = 30;

commandToSend.humidThreshold = 60;

commandToSend.manualServo = false;

drawMainLayout();

WiFi.mode(WIFI_STA);

WiFi.disconnect();

delay(100);

if (esp_now_init() != ESP_OK) {

Serial.println("ESP-NOW init failed!");

return;

}

delay(100);

esp_now_register_recv_cb(OnDataReceived);

esp_now_register_send_cb(OnDataSent);

esp_now_peer_info_t peerInfo = {};

memset(&peerInfo, 0, sizeof(peerInfo));

memcpy(peerInfo.peer_addr, senderMAC, 6);

peerInfo.channel = 0;

peerInfo.encrypt = false;

peerInfo.ifidx = WIFI_IF_STA;

if (esp_now_add_peer(&peerInfo) != ESP_OK) {

Serial.println("Failed to add peer!");

return;

}

Serial.println("ESP32 #2 ready!");

}

// =====================

// LOOP

// =====================

void loop() {

bool upPressed = readButton(BTN_UP);

bool downPressed = readButton(BTN_DOWN);

bool enterPressed = readButton(BTN_ENTER);

// ---- MAIN SCREEN ----

if (currentScreen == MAIN) {

if (newDataReceived) {

updateMainValues();

newDataReceived = false;

}

if (upPressed) {

currentScreen = MENU;

menuIndex = 0;

drawMenu();

}

}

// ---- MENU ----

else if (currentScreen == MENU) {

if (upPressed) {

menuIndex--;

if (menuIndex < 0) menuIndex = menuCount - 1;

drawMenu();

}

if (downPressed) {

menuIndex++;

if (menuIndex >= menuCount) menuIndex = 0;

drawMenu();

}

if (enterPressed) {

if (menuIndex == 0) {

currentScreen = SET_TEMP;

adjustValue = commandToSend.tempThreshold;

drawThresholdScreen("SET TEMP THRESH", adjustValue, "C");

} else if (menuIndex == 1) {

currentScreen = SET_HUMID;

adjustValue = commandToSend.humidThreshold;

drawThresholdScreen("SET HUMID THRESH", adjustValue, "%");

} else if (menuIndex == 2) {

currentScreen = SERVO_CONTROL;

drawServoScreen();

} else if (menuIndex == 3) {

currentScreen = MAIN;

drawMainLayout();

}

}

}

// ---- SET TEMP ----

else if (currentScreen == SET_TEMP) {

if (upPressed) {

adjustValue++;

if (adjustValue > 50) adjustValue = 50;

updateThresholdValue(adjustValue, "C");

}

if (downPressed) {

adjustValue--;

if (adjustValue < 0) adjustValue = 0;

updateThresholdValue(adjustValue, "C");

}

if (enterPressed) {

commandToSend.tempThreshold = adjustValue;

esp_now_send(senderMAC, (uint8_t *)&commandToSend, sizeof(commandToSend));

Serial.print("New temp threshold: ");

Serial.println(commandToSend.tempThreshold);

currentScreen = MAIN;

drawMainLayout();

}

}

// ---- SET HUMID ----

else if (currentScreen == SET_HUMID) {

if (upPressed) {

adjustValue++;

if (adjustValue > 100) adjustValue = 100;

updateThresholdValue(adjustValue, "%");

}

if (downPressed) {

adjustValue--;

if (adjustValue < 0) adjustValue = 0;

updateThresholdValue(adjustValue, "%");

}

if (enterPressed) {

commandToSend.humidThreshold = adjustValue;

esp_now_send(senderMAC, (uint8_t *)&commandToSend, sizeof(commandToSend));

Serial.print("New humid threshold: ");

Serial.println(commandToSend.humidThreshold);

currentScreen = MAIN;

drawMainLayout();

}

}

// ---- SERVO CONTROL ----

else if (currentScreen == SERVO_CONTROL) {

if (enterPressed) {

commandToSend.manualServo = !commandToSend.manualServo;

esp_now_send(senderMAC, (uint8_t *)&commandToSend, sizeof(commandToSend));

Serial.print("Servo: ");

Serial.println(commandToSend.manualServo ? "OPEN" : "CLOSED");

drawServoScreen();

}

if (upPressed) {

currentScreen = MAIN;

drawMainLayout();

}

}

esp_now_send(senderMAC, (uint8_t *)&commandToSend, sizeof(commandToSend));

delay(50);

}

Appendix C — System Photographs